Crop Protection

BASF SE, 67114 Limburgerhof, Deutschland

4630/m FIPRONIL 581 CIPAC 581, HPLC method

by

Dr. Christoph Randt and Brigitte Przywara

BASF SE

BASF Agricultural Center Limburgerhof

Crop Protection - Global Analytics

D-67114 Limburgerhof

GERMANY

- FIPRONIL

- 581

ISO common name	Fipronil

Chemical name	(±)- 5-Amino-1-[2,6-dichloro-4-(trifluoromethyl)phenyl]-4-trifluoromethylsulphinylpyrazol-3-carbonitrile (IUPAC);

(±)- 5-Amino-1-[2,6-dichloro-4-(trifluoromethyl)phenyl]-4-trifluoromethyl)-sulphinyl]-1H-pyrazole-3-carbonitrile (CA)

CAS.No.	120068-37-3

Empirical formula	C12H4Cl2F6N4OS

RMM	437.2

m.p.	195.5° C – 203°C

v.p.	3.7 x 10–7  mPa  at 25° C

Solubility	In water: 2.4 mg/l at pH 5; acetone: 545.9 g/l; methanol 137.5 g/l; 	propan-2-ol: 36.2 g/l; octanol-1: 12.2 g/l; dichloromethane: 22.3 g/l; 	hexane: 28 g/l; ethyl acetate: 264.9 g/l; toluene: 3 g/l; all at 20 °C

Description	White to yellowish crystalline powder

Formulations	Ultra low volume liquid, emulsifiable concentrates, flowable 	concentrate for seed treatment, suspension concentrate; granules, 	water dispersible granules; fine granules

### Fipronil technical

*581/TC/M/-

1 Sampling. Take at least 100 g.

## 2 Identity tests

2.1 HPLC. Use the HPLC method below. The relative retention time of the Fipronil peak in the sample solution should not differ by more than 2% from that of the calibration solution.

2.2 Infrared. Prepare potassium bromide discs from the sample and from pure fipronil using about 1 mg of material and 350 mg of potassium bromide. Scan the discs from 4000 to 400 cm-1. The spectrum produced from the sample should not differ significantly from that of the standard (Fig. 18).

- 3 Fipronil

OUTLINE OF METHOD. Fipronil is determined by high performance liquid chromatography on a reversed phase column with UV detection using the external standard method.

- REAGENTS

- Water HPLC grade

- Fipronil analytical standard of known purity

Acetonitrile HPLC grade

Propan-2-ol HPLC-grade

Mobile phase water – acetonitrile, 35 + 65 (v/v)

Calibration solution Weigh in duplicate (to the nearest 0.1 mg) about 25 mg (s mg) of fipronil standard into separate volumetric flasks (100 ml). Add propan-2-ol into each flask and place the flasks in an ultrasonic bath for approximately 10 min. Dilute to volume with propan-2-ol and mix thoroughly (solutions C1 and C2).

###### APPARATUS

Liquid chromatograph equipped with a constant flow pump, an automatic injector, a column oven, a UV detector capapbe of measuring at 280 nm

Integrator or data acquisition system

Column Nucleosil C18 (5 µm); 250 x 4 mm (i. d) or equivalent

##### Ultrasonic bath

* CIPAC method 2000. Prepared by the French Committee (CFAPA). Chairman: B. Declercq. Based on a method supplied by Rhône-Poulenc Agro, France.

PROCEDURE

(a) Operating conditions (typical)

Flow rate 			1 ml/min

Column temperature			40 °C

Injection volume				5 µl

Retention time			about 6 min

Run time			30 min

- (b) Preparation of the sample Weigh (to the nearest 0.1 mg) 25 ± 5 mg (w mg) of technical Fipronil into a volumetric flask (100 ml). Add propan-2-ol to the flask and place the flask in an ultrasonic bath for approximately 10 min. Dilute to volume with propan-2-ol and mix thoroughly (Solution S).

(c) Determination. Inject 5 µl of the calibration solution C1 until the consecutive relative response factors agree within 1%. Then, inject the calibration and sample solutions in the following sequence:

C1, S, S, C2,…

- (d) Calculation. Calculate the relative response factors (f1, f2) for the pair of calibration injections that bracket the sample injections and obtain the mean response factor f.

f=s×PHs

where:

Hs	= area of the Fipronil peak in the calibration solution

s	= mass of Fipronil standard in the calibration solution (mg)

P	= purity of Fipronil standard (g/kg)

Calculate the fipronil content for each sample injection, and calculate the mean of the two values as the fipronil content in the sample.

- Content of fipronil =f×Hww (g/kg)

where:

Hw	= area of the Fipronil peak in the sample solution

f    = mean response factor

w    = mass of sample in the sample solution (mg)

Repeatibility r	=	24 g/kg at 957 g/kg active ingredient content

Reproducibility R 	=	31 g/kg at 957 g/kg active ingredient content

- Fipronil FLOWABLE CONCENTRATE FOR SEED TREATMENT

- *581/FS/M/-

1 Sampling. Take at least 100 ml.

## 2 Identity tests

2.1 HPLC. As for Fipronil technical 581/TC/M/2.1

2.2 UV spectrometry. Use a diode array detector and measure at the top of the fipronil peak, wavelength 190 – 400 nm. The spectra of the sample and the reference have to be identical.

- 3 Fipronil

- As for technical 581 TC/M/3 except:

- Calibration solution. Calculate the amount of Fipronil, analytical standard of known purity, suitable to the expected concentration in the sample solution. For example, weigh in duplicate (to the nearest 0.1 mg) about 10 mg or 50 mg of Fipronil, depending on the concentration of the active ingredient in the formulation, into a volumetric flask (100 ml). Add 60 ml propan-2-ol. Sonicate for 10 minutes at ambient temperature. Add 10 ml of water and approx. 20 ml propan-2-ol and mix carefully. Equilibrate to ambient temperature, fill up to volume with propan-2-ol and mix thoroughly (solutions C1 and C2).

- (b) Preparation of the sample Weigh (to the nearest 0.1 mg) a calculated amount of sample corresponding to the expected amount of Fipronil, e. g. 200 mg, into a volumetric flask (100 ml). Add 10 ml of water and mix carefully. Add 60 ml propan-2-ol and mix again. Sonicate for 10 minutes at ambient temperature. Add approx. 20 ml of propan-2-ol, equilibrate to ambient temperature, fill up to volume with propan-2-ol, mix thoroughly and filtrate if necessary (Solution S).

- Fipronil SUSPENSION CONCENTRATE

- *581/SC/M/-

1 Sampling. Take at least 100 ml.

## 2 Identity test

2.1 HPLC. As for Fipronil technical 581/TC/M/2.1

2.2 UV spectrometry. As for Fipronil 581/FS/M/2.2

- 3 Fipronil

- As for 581 FS/M/3

* CIPAC method 2000. Prepared by the French Committee (CFAPA). Chairman: B. Declercq. Based on a method supplied by Rhône-Poulenc Agro, France.

- Fipronil WATER DISPERSIBLE GRANULES

- *581/WG/M/-

1 Sampling. Take at least 100 g.

## 2 Identity tests

2.1 HPLC. As for Fipronil technical 581/TC/M/2.1

2.2 UV spectrometry. As for Fipronil 581/FS/M/2.2

- 3 Fipronil

- As for 581 FS/M/3 except

- (b) Preparation of the sample Weigh (to the nearest 0.1 mg) a calculated amount of sample corresponding to the expected amount of Fipronil, e.g. 60 mg into a volumetric flask (100 ml). Add 10 ml of water and mix carefully. Add 60 ml propane-2-ol, sonicate for 10 minutes at ambient temperature. Add approx. 20 ml of propan-2-ol, equilibrate to ambient temperature, fill up to volume with propan-2-ol, mix thoroughly and filtrate if necessary (Solution S).

- Fipronil GRANULES

- *581/GR/M/-

1 Sampling. Take at least 100 g.

## 2 Identity test

2.1 HPLC. As for Fipronil technical 581/TC/M/2.1

2.2 UV spectrometry. As for Fipronil 581/FS/M/2.2

- 3 Fipronil

- As for 581 WG/M/3 except

- (b) Preparation of the sample Weigh (to the nearest 0.1 mg) a calculated amount of sample corresponding to the expected amount of Fipronil, e.g. 1000 mg into a volumetric flask (100 ml). Add 10 ml of water and mix carefully. Sonicate for 10 minutes at ambient temperature, add 60 ml propan-2-ol and mix again. Sonicate for 10 minutes at ambient temperature, add approx. 20 ml of propan-2-ol, equilibrate to ambient temperature, fill up to volume with propan-2-ol, mix thoroughly and filtrate if necessary (Solution S).

- Fipronil FINE GRANULES

- *581/FG/M/-

1 Sampling. Take at least 100 g.

## 2 Identity test

2.1 HPLC. As for Fipronil technical 581/TC/M/2.1

2.2 UV spectrometry. As for Fipronil 581/FS/M/2.2

* CIPAC method 2000. Prepared by the French Committee (CFAPA). Chairman: B. Declercq. Based on a method supplied by Rhône-Poulenc Agro, France.

- 3 Fipronil

- As for 581 WG/M/3 except

- (b) Preparation of the sample Weigh (to the nearest 0.1 mg) a calculated amount of sample corresponding to the expected amount of Fipronil, e.g. 500 mg into a volumetric flask (100 ml). Add 10 ml of water and mix carefully. Add 60 ml propan-2-ol. Sonicate for 10 minutes at ambient temperature, add approx. 20 ml of propan-2-ol, equilibrate to ambient temperature, fill up to volume with propan-2-ol, mix thoroughly and filtrate if necessary (Solution S).

- Fipronil EMULSIfiable CONCENTRATES

- *581/EC/M/-

1 Sampling. Take at least 50 ml.

## 2 Identity tests

2.1 HPLC. As for Fipronil technical 581/TC/M/2.1

- 3 Fipronil

- As for technical 581 TC/M/3

* CIPAC method 2000. Prepared by the French Committee (CFAPA). Chairman: B. Declercq. Based on a method supplied by Rhône-Poulenc Agro, France.

Typical Chromatograms of Fipronil

Figure 1 Analytical Standard Fipronil

Figure 2 Technical Material TC

Figure 3 Flowable Concentrate for Seed Treatment FS

Figure 4 Suspension Concentrate SC

Figure 5 Water Dispersible Granules WG

Figure 6 Granules GR

Figure 7 Fine Granules FG

### Fipronil EMULSIFIABLE CONCENTRATE

*581/EC/M/-

1 Sampling. Take at least 100 g.

## 2 Identity tests

2.1 HPLC. As for 581/TC/M/2.1.

2.2 Mass spectrometry. Dissolve about 500 µl of sample in propan-2-ol. Inject 5 µl of the solution through the APCI flown with a water - acetonitrile, 50 + 50 (v/v) at 0.2 ml/min and operated with a 20 V cone. Scan from 100 to 800 m.a.u. in 2 s. The following three fragments should be observed:

m/z = 435 [(M - H)- with two 35Cl]

m/z = 366 {[(M - H) - CF3]- with two 35Cl]}

m/z = 331 {[(M - H) - CF3 - Cl]- with one 35Cl]}

- 3 Fipronil

- As for 581/TC/M/3 except:

(b) Preparation of sample. Weigh (to the nearest 0.1 mg) sufficient sample to contain about 25 mg (w mg) of fipronil into a volumetric flask (100 ml). Add propan-2-ol into the flask and place the flask in an ultrasonic bath for approximately 10 min. Dilute to volume with propan-2-ol and mix thoroughly (solution S).

Repeatability r	= 1.0 g/kg at 42.4 g/kg active ingredient content

Reproducibility R	= 1.6 g/kg at 42.4 g/kg active ingredient content

### Fipronil ULTRA LOW VOLUME LIQUID

*581/UL/M/-

1 Sampling. Take at least 100 g.

## 2 Identity tests

2.1 HPLC. As for 581/TC/M/2.1.

2.2 Mass spectrometry. As for 581/EC/M/2.2.

* CIPAC method 2000. Prepared by the French Committee (CFAPA). Chairman: B. Declercq. Based on a method supplied by Rhône-Poulenc Agro, France.

- 3 Fipronil.

- As for 581/TC/M/3 except:

(b) Preparation of sample. Weigh (to the nearest 0.1 mg) sufficient sample to contain about 25 mg (w mg) of fipronil into a volumetric flask (100 ml). Add propan-2-ol into the flask and place the flask in an ultrasonic bath for approximately 10 min. Dilute to volume with propan-2-ol and mix thoroughly (solution S).

Repeatability r	= 1.5 g/kg at 25.8 g/kg active ingredient content

Reproducibility R	= 1.8 g/kg at 25.8 g/kg active ingredient content

Repeatability r	= 0.2 g/kg at 6.8 g/kg active ingredient content

Reproducibility R	= 0.6 g/kg at 6.8 g/kg active ingredient content

Figure 8 Chromatogram of fipronil

Figure 9 IR spectrum of fipronil

*

*

*

*

*

*

*

*

*