CYPRODINIL

Collaborative Study

Full Scale Collaborative Study 
for the 
Determination of 
Cyprodinil Active Ingredient and Formulations by 
High Performance Liquid Chromatography

Report to CIPAC

by

Syngenta Crop Protection AG

Siegfried Völlmin

Im Breitenloh 5

CH-4333 Münchwilen

Switzerland

May 2008

Content 	page

1.	Participants	3

2.	Active Ingredient, General Information	5

3.	Samples	6

4.	Method	6

4.1	Scope	6

4.2	Principle	6

4.3	Procedure	6

5.	Remarks of the Participants	7

6.	Evaluation and Discussion	8

6.1	Screening for valid data	8

6.2	Analysis of Data	8

6.3	Determination of Cyprodinil – with indication of outliers / strugglers	9

7.	Conclusions	12

8.	Appendix 1: Graphical Presentation of Results	13

# Participants

| Benešová, Karolina | State Phytosanitary Administration Dept of Chem. Laboratories Zemědělská 1A 613 00 Brno Czech Republic |
| --- | --- |
| Benke, Lajos | Agricultural Office of County Fejér Plant Protection and Soil Conservation Directorate 2481-Velence, Ország út 23 Hungary |
| Dardemann, Jörg Dr. | StählerTec Deutschland GmbH & Co. KG Stader Elbstraße 26-28 21683 Stade Deutschland |
| Dubois Alain Dr. | Laboratoire fédéral pour la Sécurité alimentaire - AFSCA Rue Boumal, 5 B-4000 LIEGE |
| Flubacher, Martin | Syngenta Crop Protection Münchwilen AG Analytical Development & Product Chemistry Martin Flubacher Im Breitenloh CH-4333 Münchwilen Schweiz |
| Garvey, Jim Dr. | Pesticida Control Laboratory Department of Agriculture, Food and Fisheries Backweston Laboratory Campus Celbridge Co. Kildare Ireland |
| Iurascu, Teodora | Central Laboratory for Phytosanitary Quarantine 11 Afumati ST, Bucharest Romania |
| Janeš , Lucija | Kmetijski inštitut Slovenije Agricultural Institute of Slovenia Hacquetova 17, 1000 Ljubljana Slovenia |
| Jivkova, Nikolinka | Central Laboratory for Chemical Testing and Control National Plant Protection Service Sofia Bulgaria |
| Krongaard, Teddy | Department of Atmospheric Environment National Environmental Research Institute University of Aarhus Frederiksborgvej 399 P.O. 358 DK-4000 Roskilde Denmark |
| Kim, Sungwoo Dr. | Pesticide Safety Division National Institute of Agricultural Science & Technology Rural Development Administration 249 Seodun-dong, Suwon, 441-707 Republic of Korea |

| Luetrakool, Nunchana | Agricultural ProdustionScience Research Dev. Office Department of Agriculture Phaholyothin Rd. Lardyao, Chatuchak Bangkok Thailand 10900 |
| --- | --- |
| Manso, Luis | Lab Arbitral Agroalimentario Ministerio de Agricultura, Pesca y Alimentación Ctra. A-6. PK 10, 700. Aravaca 28023 Madrid Spain |
| Marais Susan | Pesticide Analytical Technology cc Apple Leaf Office Park 6 Corner of Veda & Montana Street Montana Park Pretoria South Africa |
| Plumb, Andrew | Food Science Group Central Science Laboratory Sand Hutton York YO41 1LZ UK |
| Rodríguez García, Alejandro | Laboratorio Agroalimentario. Sevilla C/ Bergantín, 39 41012 Sevilla España |
| Santilio, Angela Dr. | National Institute of Health (Istituto Superiore di Sanità) Department of Environment and Primary Prevention Pesticide Unit 00161 Rome Italy |
| Unterweger, Heidrun | AGES Wien CC Rückstandsanalytik Ing. Heidrun Unterweger Spargelfeldstarße 191 A-1226 Wien Österreich |
| Vinke, Claudia Dr. | Federal Office of Consumer Protection and Food Safety Dept. Plant Protection Products Messeweg 11/12 D-38104 Braunschweig Deutschland |

Participants are listed in alphabetical sequence, lab numbers in the result tables were assigned in order of receiving the confirmation to participate on the collaborative study.

# Active Ingredient, General Information

Chemical Abstracts Name:	2-pyrimidinamine, 4-cyclopropyl-6-methyl-N-phenyl-

ISO common name:		Cyprodinil

CAS-Nr.:  			121552-61-2

Structure:

Molecular mass: 		225.3

Empirical formula:		C14H15N3

Activity:  			Fungicide

# Samples

In January 2008, Information Sheet No. 278 was sent out by the CIPAC Secretary inviting members to participate in a collaborative study on the determination of Cyprodinil by HPLC in technical grade active ingredient (TGAI) and in formulation.

The participants who completed the study are listed in section 1.

Five test samples and one analytical standard were sent to the participants:

1.	Cyprodinil Tech1

- Cyprodinil Tech2

- KAYAK 300 EC

- UNIX 75 WG

- CHORUS 50 WG

Cyprodinil analytical standard, 99.9 % (w/w) purity.

By end of April 2008, 19 of 20 respondents had returned their results.

# Method

## Scope

Determination of the content of cyprodinil in technical grade active ingredient and in formulations.

## Principle

Cyprodinil is determined by reversed phase high performance liquid chromatography (HPLC) using a Nucleosil 5μm C18 column and UV detection at 254 nm. Quantification is by external standardization.

## Procedure

Each sample was analyzed by four independent determinations. Each sample was analyzed by duplicate injections of two independent weighings on each of two different days. On the second day, both reference and test solutions were freshly prepared. For the calculation of the response factor the mean of the response factor (double injection) before and after the sample was used (bracketing). For the calculation of the content of a sample, the mean value of the duplicate injections was used.

# Remarks of the Participants

Several participants made comments about the performance of the method and noted deviations from the method:

| Laboratory 1 | Column: Phenomenex Luna C18/2 100A, 250 x 4.6 mm, flow 1.5 ml/min. |
| --- | --- |
| Laboratory 2 | Column temperature set to 30°C to keep constant analytical conditions (retention time), because ambient temperature in laboratory changed during the day. pH of the 1% v/v of the trifluoro acetic acid determined by pH-meter was 1.11. |
| Laboratory 3 | Column: Agilent Eclipse XDB-C18, 5 µm, 4.6 x 150 mm |
| Laboratory 4 | Column: Nucleodur C18 |
| Laboratory 5 | Peak tailing was observed in both the standards and the samples. Diameter of column was 4.6 mm. Method was easy to follow and very straightforward. |
| Laboratory 6 | No comments |
| Laboratory 7 | Column: Merck KGaA, LichroCART, Superspher 60, RP-select B, 250 x 4 mm |
| Laboratory 8 | Inertsil ODS-2, 4.6 x 250 mm |
| Laboratory 9 | Good peak shape. Good repeatability. It seems if the worksheet formula did not compensate for the density of the EC formulation. The final result for the EC formulation is thus m/m and not m/v. I prefer to grind a granular formulation to a fine powder before weighing the sample into a volumetric flask. This will give better repeatability. (In the case of this study, I followed the prescribed procedure, without grinding.) Flow rate was reduced from 1.2 ml/min to 1.0 ml/min and a shorter column of 150 x 4.6 mm (Zorbax Eclipse XDB-C18) was used. |
| Laboratory 10 | On day 2 the retention time of the cyprodinil in a couple of injections drifted slightly (approx. 0.1 of a minute). This was observed in the second injection of Tech 2(a) and the first injection of 75 WG(b). This resulted in larger peak areas. |
| Laboratory 11 | Due to normal laboratory practice the samples of Chorus WG and Unix WG were crushed in a mortar. |
| Laboratory 12 | Peak shape OK. Timesaving and uncomplicated Method. |
| Laboratory 13 | No comments |
| Laboratory 14 | No comments |
| Laboratory 15 | Column: Phenomenex Luna C18, 250 x 4.6 mm |
| Laboratory 16 | Column is different (4 micrometers, 3.9 x 150 mm), so the retention time is shorter. Equilibration time is approximately one hour. With 1% aqueous trifluoroacetic acid the pH is about 1.2, and the limited range of the pH for the column is 2. With 0.05% TFA the pH is 2.25. (We have chosen this pH because cyprodinil pKa is 4.4). |
| Laboratory 17 | A 150 mm liquid chromatographic column was used. A 0.20µm filter was used to clear the suspension of the EC and WG formulation. |
| Laboratory 18 | Flow rate: 1.0 ml/min. |
| Laboratory 19 | We did a little bit change of the mobile phase: We decreased the amount of the TFAA in water – 0.5% instead of 1%, because pH was very low and we were afraid for the column. It was made 1.8. Column: Merck, Purospher star C18 endcapped, LiCroCART 250-4.6, 5 µm. |

# Evaluation and Discussion

## Screening for valid data

The data obtained from each laboratory were reviewed to determine if there had been any significant deviations which might have affected the analytical results.

Some labs (nos.3, 9, 16, 17) used shorter columns (150 mm) and some labs (nos.16, 19) changed the 1% TFAA to 0.5% in the mobile phase. These changes as well as other small deviations noted by the participants were not expected to affect the analytical results.

Visual examination of the chromatograms showed, that no problems had been experienced during the analysis – whether an alteration had been made or not.

Therefore it was decided to take the data of all laboratories for the statistical evaluation.

## Analysis of Data

Results reported by the laboratories and the statistical evaluation are listed in tables 1-3 and displayed in figures 1-5.

The statistical evaluation of the data was performed following the “Guidelines for CIPAC Collaborative Study Procedures for Assessment of Performance of Analytical Methods”, according to DIN ISO 5725. The data were examined for outliers and strugglers using Cochran’s test on the within-lab variance, followed by Grubb’s test on the lab means (between lab variance), and iterating where necessary. The tests were performed at an alpha level of 0.01 for outlier, and 0.05 for struggler.

Based on this procedure, the Cochran variance homogeneity test identified one outlier each for the Tech2 (Lab 16) and the Chorus WP50 formulation (Lab 12). Nevertheless the results of these outliers have been used for the calculations in table 3.

The Grubb’s test on the lab means identified one outlier each for the Tech1 (Lab 19), Kayak 300 EC (Lab 2), Unix 75WG (Lab 2) and Chorus 50 WG (Lab 2). Two outliers were identified for Tech2 (Lab 16 and 19). For Kayak 300 EC also a struggler (Lab16) was identified.

Evaluation of RSD according to the Horwitz criteria showed that the repeatability is within the accepted range without elimination of any of the outliers or strugglers.

## Determination of Cyprodinil – with indication of outliers / strugglers

Table 1 Individual Results – all values in g/kg

|  | Cyprodinil tech I |  | Cyprodinil tech II |  | KAYAK 300 EC |  | UNIX 75 WG |  | CHORUS 50WG |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Day1 | Day2 | Day1 | Day2 | Day1 | Day2 | Day1 | Day2 | Day1 | Day2 |
| Laboratory 1 | 985.2 | 986.9 | 983.2 | 987.0 | 297.6 | 295.2 | 749.1 | 748.7 | 501.7 | 501.4 |
| Laboratory 2 | 987.7 | 990.7 | 990.1 | 992.0 | 286.6 | 284.3 | 716.7 | 719.6 | 479.5 | 483.6 |
| Laboratory 3 | 998.1 | 990.1 | 995.1 | 993.9 | 296.6 | 296.6 | 752.0 | 749.9 | 507.1 | 508.6 |
| Laboratory 4 | 997.1 | 995.3 | 996.5 | 997.5 | 292.5 | 295.2 | 744.4 | 745.8 | 499.3 | 502.0 |
| Laboratory 5 | 994.4 | 1002.6 | 996.0 | 1000.4 | 297.2 | 299.2 | 741.9 | 742.7 | 502.2 | 491.6 |
| Laboratory 6 | 991.2 | 991.5 | 992.0 | 998.4 | 298.2 | 299.7 | 749.6 | 750.4 | 500.4 | 504.3 |
| Laboratory 7 | 991.5 | 981.9 | 988.9 | 998.7 | 292.8 | 293.4 | 738.9 | 736.3 | 498.0 | 498.5 |
| Laboratory 8 | 992.8 | 987.9 | 995.9 | 989.8 | 294.9 | 296.2 | 741.2 | 741.9 | 496.9 | 498.3 |
| Laboratory 9 | 994.4 | 996.0 | 992.4 | 989.4 | 297.8 | 293.3 | 754.0 | 750.0 | 502.6 | 507.8 |
| Laboratory 10 | 986.4 | 984.9 | 984.8 | 998.0 | 296.9 | 301.9 | 742.3 | 767.6 | 500.7 | 502.7 |
| Laboratory 11 | 1002.2 | 994.8 | 991.5 | 998.1 | 294.1 | 296.3 | 751.6 | 744.9 | 498.8 | 504.9 |
| Laboratory 12 | 985.3 | 1009.6 | 987.0 | 1009.8 | 298.4 | 281.7 | 728.5 | 755.7 | 491.8* | 521.5* |
| Laboratory 13 | 992.4 | 992.1 | 993.7 | 993.3 | 292.5 | 292.8 | 748.2 | 741.9 | 514.2 | 515.6 |
| Laboratory 14 | 994.2 | 995.9 | 996.9 | 999.9 | 295.4 | 290.7 | 748.8 | 746.0 | 500.6 | 501.9 |
| Laboratory 15 | 990.0 | 1000.4 | 985.8 | 999.7 | 293.1 | 298.8 | 740.1 | 751.7 | 500.1 | 500.9 |
| Laboratory 16 | 992.0 | 998.5 | 944.0* | 1006.0* | 294.2 | 312.6 | 752.2 | 747.0 | 506.4 | 503.9 |
| Laboratory 17 | 999.4 | 1000.1 | 999.4 | 998.7 | 295.1 | 296.4 | 744.5 | 743.1 | 500.3 | 504.3 |
| Laboratory 18 | 1009.4 | 990.4 | 1008.7 | 992.4 | 300.0 | 295.7 | 733.3 | 748.1 | 502.5 | 502.1 |
| Laboratory 19 | 974.5 | 969.3 | 973.8 | 959.7 | 295.4 | 290.7 | 746.3 | 740.5 | 492.7 | 490.6 |

* Cochran outlier

Table 2 Mean values

|  | Cyprodinil - tech I | Cyprodinil - tech II | KAYAK 300 EC | UNIX 75 WG | CHORUS 50WG |
| --- | --- | --- | --- | --- | --- |
| Laboratory 1 | 986.1 | 985.1 | 296.4 | 748.9 | 501.6 |
| Laboratory 2 | 989.2 | 991.1 | 285.5* | 718.2* | 481.6* |
| Laboratory 3 | 994.1 | 994.5 | 296.6 | 751.0 | 507.9 |
| Laboratory 4 | 996.2 | 997.0 | 293.9 | 745.1 | 500.7 |
| Laboratory 5 | 998.5 | 998.2 | 298.2 | 742.3 | 496.9 |
| Laboratory 6 | 991.4 | 995.2 | 299.0 | 750.0 | 502.4 |
| Laboratory 7 | 986.7 | 993.8 | 293.1 | 737.6 | 498.3 |
| Laboratory 8 | 990.4 | 992.9 | 295.6 | 741.6 | 497.6 |
| Laboratory 9 | 995.2 | 990.9 | 295.6 | 752.0 | 505.2 |
| Laboratory 10 | 985.7 | 991.4 | 299.4 | 755.0 | 501.7 |
| Laboratory 11 | 998.5 | 994.8 | 295.2 | 748.3 | 501.9 |
| Laboratory 12 | 997.5 | 998.4 | 290.1 | 742.1 | 506.7 |
| Laboratory 13 | 992.3 | 993.5 | 292.7 | 745.1 | 514.9 |
| Laboratory 14 | 995.1 | 998.4 | 293.1 | 747.4 | 501.3 |
| Laboratory 15 | 995.2 | 992.8 | 296.0 | 745.9 | 500.5 |
| Laboratory 16 | 995.3 | 975.0* | 303.4** | 749.6 | 505.2 |
| Laboratory 17 | 999.8 | 999.1 | 295.8 | 743.8 | 502.3 |
| Laboratory 18 | 999.9 | 1000.6 | 297.9 | 740.7 | 502.3 |
| Laboratory 19 | 971.9* | 966.8* | 293.1 | 743.4 | 491.7 |
| Overall Mean | 992.6 | 992.1 | 295.3 | 744.6 | 501.1 |

* Grubbs outlier

** Grubbs struggler

Table 3

Summary of the statistical evaluation - no elimination of any outliers / strugglers

|  | Cyprodinil Tech 1 | Cyprodinil Tech 2 | KAYAK 300 EC | UNIX 75 WG | CHORUS 50 WG |
| --- | --- | --- | --- | --- | --- |
| Xm [g/kg] | 992.6 | 992.1 | 295.5 | 744.6 | 501.1 |
| L | 19 | 19 | 19 | 19 | 19 |
| Sr | 6.19 | 12.00 | 4.50 | 7.12 | 5.47 |
| SL | 6.42 | 8.18 | 3.74 | 7.67 | 6.67 |
| SR | 8.92 | 14.52 | 5.85 | 10.47 | 8.63 |
| r | 17.32 | 33.60 | 12.60 | 19.94 | 15.32 |
| R | 24.97 | 40.66 | 16.39 | 29.31 | 24.15 |
| RSDr | 0.62 | 1.21 | 1.52 | 0.96 | 1.09 |
| RSDR | 0.90 | 1.46 | 1.98 | 1.41 | 1.72 |
| RSDR(Hor) | 2.00 | 2.00 | 2.40 | 2.09 | 2.22 |

Horwitz limits fulfilled for all test samples

xm		=	overall sample mean

L		=	number of laboratories

sr 		=	repeatability standard deviation

RSDr		=	relative repeatability standard deviation

r		=	repeatability limit

sR		=	reproducibility standard deviation

RSDR		=	relative reproducibility standard deviation

R		=	reproducibility limit

sL		=	“pure” between laboratory standard deviation

RSDR(Hor)	=	relative reproducibility standard deviation (Horwitz equation)

# Conclusions

In this collaborative study 19 different laboratories out of 20 original respondents participated and returned results. One laboratory did not send results within the required timescale.

The results from all participants have been included in the calculations, including the outliers and strugglers.

As shown in the statistical summary given in Table 3, for all samples the between laboratory experimental relative reproducibility standard deviation (% RSDR) is well below the calculated acceptable value (% RSDR (Hor)) based on the Horwitz curve calculation.

Therefore we consider this method to be suitable without further changes and recommend accepting it as a provisional CIPAC method for the determination of Cyprodinil in TGAI and formulations WP and EC.

# Appendix 1: Graphical Presentation of Results

Figure 1: Cyprodinil Tech1

Figure 2: Cyprodinil Tech2

Figure  3: KAYAK 300EC

Figure 4: UNIX 75WG

Figure 5: CHORUS 50WG