HALOXYFOP-P-METHYL

526

ISO common name	Haloxyfop-P-methyl

Chemical name	Methyl (R)-2-[4-(3-chloro-5-trifluoromethyl-2-pyridyloxy)phenoxy]propionate (IUPAC); (+)-2-[4-[[3-chloro-5-(trifluoromethyl)-2-pyridinyl]oxy] phenoxy]propanoic acid methyl ester (CA; 72619-32-0)

Empirical formula	C16H13ClF3NO4

RMM	375.7

b.p.	> 280C

v.p.	2.6  10-5 Pa at 20°C

Solubility	In water 9.08 mg/l at 20C.

In most organic solvents > 1000 g/l at 20C.

Description	Light amber, viscous liquid

Stability	Unstable in alkaline conditions

Formulations	Emulsifiable Concentrate (EC)

HALOXYFOP-P-METHYL TECHNICAL

526/TC/(M)/-

1	Sampling.  Take at least 50 g.

2	Identity tests.

2.1	HPLC.  Use the HPLC method below.  The retention time of haloxyfop-P-methyl for the sample solution should not deviate by more than 2% from that of the calibration solution.

2.2	UV spectrum.  Use the HPLC method 526/TC/(M)/3 and record the spectrum of the haloxyfop-P-methyl peak with a diode array detector.  The spectrum obtained from the sample should not differ significantly from that of the standard (Fig. 1).

2.3	IR.  Prepare potassium bromide discs from the sample and from pure haloxyfop-P-methyl.  Scan the discs from 4000 – 600 cm-1.  The spectrum produced from the sample should not differ significantly from that of the standard (Fig. 2).

3	Haloxyfop-P-methyl.

OUTLINE OF METHOD Haloxyfop-P-methyl (R-enantiomer) is separated from the S-enantiomer and other components and is determined by normal phase high performance liquid chromatography on a chiral phase column using UV detection at 280 nm and external standardization.

REAGENTS

Haloxyfop-P-methyl of known purity.  Store refrigerated.

Heptane

Propan-2-ol

Methanol

Dilution solvent.  Heptane – propan-2-ol, 80 + 20, (v/v)

Mobile phase.  Heptane – propan-2-ol – methanol, 85 + 5 + 10, (v/v)

Calibration solution. Prepare calibration solutions in duplicate.  Weigh (to the nearest 0.1 mg) about 130 mg (s mg) of haloxyfop-P-methyl directly into a wide mouth bottle (100 ml).  Add by pipette dilution solvent (50.0 ml), place the bottle on a mechanical shaker for 5 min and mix.

APPARATUS

Liquid chromatograph equipped with a variable-wavelength UV detector operated at 280 nm.

Liquid chromatographic column, 250  4.6 (i.d.) mm, packed with Chiralcel OK with 10 µm particles (Chiral Technologies, Inc., No. 18025), or equivalent.

Integrator or data system

Mechanical shaker

PROCEDURE

(a)	Operating conditions (typical)

Column	250  4.6 (i.d.) mm, packed with Chiralcel OK with 10 µm particles

Mobile Phase	heptane – propan-2-ol – methanol, 85 + 5 + 10 (v/v)

Flow rate	1.8 ml/min

Temperature	25C

Detector Wavelength	280 nm

Injection Volume	10 µl

Run duration	15 minutes

Retention times	haloxyfop-P-methyl: about 4.5 min

S-enantiomer: about 6.0 min

(b)	Sample preparation.  Weigh (to the nearest 0.1 mg) in duplicate directly into two wide mouth bottles (100 ml) about 130 mg (w mg) of haloxyfop-P-methyl.  Add by pipette dilution solvent (50.0 ml), place the bottle on a mechanical shaker for 5 min and mix.

(c)	System Equilibration.  Pump sufficient eluant through the column to equilibrate the system.  Inject 10 µl of calibration solution into the column and adjust the operating conditions to give retention times about 4.5 and 6.0 min for haloxyfop-P-methyl and S-enantiomer respectively.  Resolution between the enantiomers should be 1.5.  Inject 10 µl portions of the calibration solution until the response factors obtained for two consecutive injections differ by less than 1.0%.  Then inject 10 µl of a second calibration solution.  The response factor for this solution should not deviate by more than 1.0% from that for the first calibration solution, otherwise prepare new calibration solutions.

(d)	Determination.  Inject in duplicate 10 µl portions of the sample solution bracketing them by injections of the calibration solutions as follows:  calibration solution CA, sample solution SA, sample solution SA, calibration solution CB, sample solution SB, sample solution SB, calibration solution CA, and so on.  Measure the relevant peak areas.

(e)	Calculation.  Calculate the mean value of each pair of response factors bracketing the two injections of a sample and use this value for calculating the haloxyfop-P-methyl contents of the bracketed sample injections.

fi=s×PHs

Content of haloxyfop-P-methyl =f×Hww g/kg

where:

fi	=	individual response factor

f	=	mean response factor

Hs	=	peak area of haloxyfop-P-methyl in the calibration solution

Hw	=	peak area of haloxyfop-P-methyl in the sample

s	=	mass of haloxyfop-P-methyl working standard in the calibration solution (mg)

w	=	mass of the sample taken (mg)

P	=	purity of haloxyfop-P-methyl working standard (g/kg)

Repeatability r	=	28 g/kg at 972 g/kg active ingredient content

29 g/kg at 973 g/kg active ingredient content

Reproducibility R	=	37 g/kg at 972 g/kg active ingredient content

36 g/kg at 973 g/kg active ingredient content

HALOXYFOP-P-METHYL EMULSIFIABLE CONCENTRATES

526/EC/(M)/-

1	Sampling.  Take at least 250 g.

2	Identity tests.

2.1	HPLC.  As for 526/TC/(M)/2.1

2.2	UV spectrum.  As for 526/TC/(M)/2.2

3	Haloxyfop-P-methyl.  As for 526/TC/(M)/3 except:

(b)	Sample preparation.  Weigh (to the nearest 0.1 mg) in duplicate directly into two volumetric flasks (50 ml) sufficient sample to contain about 130 mg (w mg) of haloxyfop-P-methyl.  Make up to volume with dilution solution and mix well.

Repeatability r	=	2.0 g/kg at 36.1 g/kg active ingredient content

5.7 g/kg at 106 g/kg active ingredient content

14 g/kg at 462 g/kg active ingredient content

Reproducibility R	=	3.1 g/kg at 36.1 g/kg active ingredient content

7.4 g/kg at 106 g/kg active ingredient content

22 g/kg at 462 g/kg active ingredient content

Fig. 1  UV spectrum of haloxyfop-P-methyl

Fig. 2  IR spectrum of haloxyfop-P-methyl