521. Fluazinam

HPLC methods

CIPAC Collaborative Trial
according to 
CIPAC Information Sheet No 279

##### Sarah Stiénon,

ISK Biosciences Europe S.A.
B-1050 brussels

- Belgium

##### October 2009

### FLUAZINAM

521

- ISO common name:	Fluazinam

Chemical name:   	3-chloro-N-[3-chloro-2,6-dinitro-4-(trifluoromethyl)phenyl]-5-(trifluoromethyl)-2-pyridinamine

CAS-Number:  	79622-59-6

RMM: 	465.1

Empirical formula:	C13 H4 Cl2 F6 N4 O4

m.p.	115 – 117 °C

V.p.	7.5 mPa at 20°C

Solubility	In water : 0.135 mg/l at pH 7 and at 20°C
In n-hexane : 8 g/l at 25°C
In acetone : 853 g/l at 25°C
In dichloromethane : 675 g/l at 25°C
In methanol : 192 g/l at 25°C

Stability	Avoid exposure to heat and light.

Hydrolysis	DT50 3.6 d (pH 7), 3.7 d (pH 9) and stable at pH 5

Description	Form : Light yellow crystals

Formulation	Suspension concentrate

### FLUAZINAM technical

521/TC/M/-

1 Sampling. Take at least 20 g.

## 2 Identity test

2.1 HPLC. Use the reversed phase HPLC method 3.1 described below. The relative retention time of the fluazinam peak in the sample solution should not deviate by more than 2% from that of the calibration solution. The UV spectrum measured from this peak should match that obtained from the calibration substance.

- 3 Fluazinam Outline of the Method

Fluazinam is determined by reversed phase high performance liquid chromatography using UV detection at 240 nm and external standard calibration.

Determination of Fluazinam by reversed phase HPLC

- REAGENTS

- Fluazinam reference standard of known content

Water HPLC grade

Acetonitrile HPLC grade

Triethylamine, analytical reagent grade

o-phosphoric acid (H3PO4), 85%, analytical reagent grade

- Eluent A: 0.2 % (v/v) aqueous solution with triethylamine, pH adjusted to 3.0 with o-phosphoric acid

- Eluent B: acetonitrile

Solvent :acetonitrile

Calibration solutions. Weigh in duplicate (to the nearest 0.1 mg) 50 mg of the fluazinam reference standard (s mg) into separate volumetric flasks (50 ml). Add acetonitrile (about 40 ml) and place the flasks in an ultrasonic bath until complete dissolution (minimum for 5 minutes). Allow the solutions to cool to ambient temperature and fill to the mark with acetonitrile (stock calibration solutions C1 and C2). Mix well.

Transfer by pipette 5, 10 and 15 mL (t mL) of stock calibration solution C1 into separate volumetric flasks (50 mL) and fill to the mark with acetonitrile (calibration solutions C1A, C1B and C1C).

Transfer by pipette 10 mL (t mL) of stock calibration solution C2 into a separate volumetric flask (50 mL) and fill to the mark with acetonitrile (calibration solutions C2B).

###### APPARATUS

High performance liquid chromatograph equipped with a detector suitable for operation at 240 nm (UV-detection) an oven and an injection system capable of injecting 10 µl

Liquid chromatographic column stainless steel, 250 x 4.6 mm (i.d), packed with Cosmosil C18, 
5 µm, Phenomenex or equivalent with the same selectivity

Electronic integrator or data system

Ultrasonic bath

pHmeter

PROCEDURE

(a) Chromatographic conditions (typical)

Column temperature	50°C

Flow rate	0.75 ml/min

Detector wavelength	240 nm

Injection volume	10 µl

Retention time	approximately 36 min

Gradient programme		Time	Eluent A	Eluent B

[ min ]	[ % ]	[ % ]

0	38	62

29	38	62

35	25	75			45	25	75

60	38	62

65	38	62

(b) Equilibration of the system. Pump sufficient mobile phase (use gradient programme) through the column to equilibrate the system. Inject 10 l portions of the calibration solution C1B and repeat the injections until retention times and peak areas deviate by less than  0.5 % from the mean for three successive injections.

- (c) Sample preparation. Prepare sample solutions in duplicate for each sample. Weigh (to the nearest 0.1 mg) sufficient sample (w mg) to contain about 120 mg fluazinam into a volumetric flask (50 ml). Add acetonitrile (about 40 ml) and place the flasks in an ultrasonic bath until complete dissolution (minimum for 5 minutes). Allow the solutions to cool to ambient temperature and fill to the mark with acetonitrile (sample solutions S1 and S2). Mix thoroughly. Transfer by pipette 4 ml of this solution with acetonitrile and fill to the mark with acetonitrile to a volume of 50 ml. Filter a portion of each standard solution through a 0.45 µm fitter prior to analysis.

(d) Determination. Inject 10 l portions of the second calibration solution (C2B) for two successive injections. The mean response factor for this solution should deviate by no more than 1% from those for the first calibration solution (C1B) (see paragraph (b) Equilibration of the system), otherwise the calibration solutions should be prepared again.

Inject in duplicate 10 l portions of each sample solution (S1, S2, …,etc.) bracketing them by single injections of the three calibration solutions (C1A, C1B and C1C) using the following sequence:

- C1A, S1, S1, S2, S2, C1B, S3, S3, S4, S4, C1c…

Determine the peak area of fluazinam

- (e) Preparation of a calibration curve

- Prepare a calibration curve for fluazinam by plotting peak areas versus the concentration of fluazinam (m µg/ml) in the three calibration solutions. Using the method of least-squares calculate the equation for the straight line that best fits the experimental data. The correlation coefficient should be 0.999 or better. If not repeat the calibration.

- n=s×PDc

where:

n	=	concentration of fluazinam in diluted calibration solution (µg/ml)

P	= purity of fluazinam reference standard (g/kg)

s	= mass of fluazinam reference standard in the stock calibration solution (mg)

Dc	= dilution factor of calibration solution.

- (e) Calculation

- Determine the content of fluazinam of the sample solutions for each sample injection.

- fluazinam content =(Hw− b)a x w x 625 [g/kg] (M)

where:

Hw	= peak area of fluazinam in the sample solution

b	= intercept of calibration curve

a	= slope of calibration curve

w	= mass of sample taken (mg)

###### Typical Chromatograms 521/TC/M/3

FLUAZINAM SUSPENSION CONCENTRATES

- 521/SC/M/-

1 Sampling. Take at least 100 g.

2 Identity test

2.1 HPLC. As for fluazinam technical 521/TC/M/2.1

#### 3 Fluazinam

Same approach as for fluazinam technical 521/TC/M/3

#### 3.1 Determination of Fluazinam by reversed phase HPLC
As for fluazinam technical 521/TC/M/3 except

- (c) Sample preparation. Prepare sample solutions in duplicate for each sample. Weigh (to the nearest 0.1 mg) sufficient sample (w mg) to contain about 50 to 70 mg fluazinam into a volumetric flask (25 ml). Add acetonitrile (about 20 ml) and place the flasks in an ultrasonic bath for 5 minutes). Allow the solutions to cool to ambient temperature and fill to the mark with acetonitrile (sample solutions S1 and S2). Mix thoroughly. Transfer by pipette 4 ml of this solution with acetonitrile and fill to the mark with acetonitrile to a volume of 50 ml. Filter a portion of each standard solution through a 0.45 µm fitter prior to analysis.

- (e) Calculation

- Determine the content of fluazinam of the sample solutions for each sample injection.

- fluazinam content =(Hw− b)a x w x 312.5 [g/kg] (M)

where:

Hw	= peak area of fluazinam in the sample solution

b	= intercept of calibration curve

a	= slope of calibration curve

w	= mass of sample taken (mg)

###### Typical Chromatograms 521/SC/M/3