- SPINOSAD

- 636

- See CIPAC L, p 121.

- SPINOSAD EMULSIFIABLE CONCENTRATES

636/EC/m/-

1	Sampling.  Take at least 250 g.

2	Identity tests.

2.1	HPLC.  As for 636/TC/M/2.1

2.2	LC-MS.  As for 636/TC/M/2.2

3	Spinosad.  As for 636/TC/M/3 except:

(b)	Sample preparation.  Mix thoroughly.  Weigh (to the nearest 0.1 mg) into a flask (100 ml) sufficient sample to contain about 25 mg of spinosad (w mg).  Add 5 ml water and 95 ml methanol and mix thoroughly (Note 1).  Filter through 0.2-µm PTFE membrane.

Note 1:  Depending on the solvent used in the EC the oil might not dissolve completely in the methanol extraction solvent. This should be checked carefully in each EC formulation to be analysed.