585. Fosthiazate

HPLC method

CIPAC Small Scale Collaborative Trial

##### Frédéric Joris

ISK Biosciences Europe S.A.
B-1050 brussels

- Belgium

##### November 2010

### FOSTHIAZATE

585

- ISO common name:	Fosthiazate

Chemical name:   	O-Ethyl-S-(1-methylpropyl)(2-oxo-3-thiazolidinyl)phosphonothioate

IUPAC:

(RS)-[S-(RS)-sec-butyl O-ethyl 2-oxo-1,3-thiazolidin-3-ylphosphonothioate]
or
(3RS)-3-[(RS)-sec-butylthio(ethoxy)phosphinoyl]-1,3-thiazolidin-2-one

CAS-Number:  	98886-44-3

RMM: 	283.36

Empirical formula:	C9H18NO3PS2

m.p.	Not applicable; fosthiazate is a liquid.

V.p.	5.6 x 10-4 Pa at 25 °C (98.4 % purity)

Solubility	In water: at 25°C (99.3% purity): 9.88 g/l at pH 5, 9.00 g/l at pH 7 and 9.46 g/l at pH 9.

In organic solvents: 
In n-hexane : not solubilised 15.1g/l at 20°C (92.2% purity)
In xylene: solubilised at 20°C (92.2% purity)
In N-methyl-2-pyrrolidone: solubilised at 20°C (92.2% purity)
In isopropyl alcohol: solubilised at 20°C (92.2% purity)

Stability	Keep frozen (< -18 °C) when not in use.

Hydrolysis	DT50 at 25°C in the dark; calculated DT 50s for [14C]-butyl and         [14C]- thiazolidinone labelled fosthiazate:

| pH | B-label | T-label |
| --- | --- | --- |
| 5 | 191 days | 163 days |
| 7 | 102 days | 107 days |
| 9 | 3.2 days | 3.3 days |

Description	Form: clear liquid

Formulation		Granules (GR)

### FOSTHIAZATE technical

585/TC/M/-

1. Sampling. Take at least 20 g.

## 2. Identity test

2.1 HPLC. Use the reversed phase HPLC method 3 described below. The relative retention time of the fosthiazate peak in the sample solution should not deviate by more than 2% from that of the calibration solution. The UV spectrum measured from this peak should match that obtained from the calibration substance.

- 3. Fosthiazate

OUTLINE OF METHOD. Fosthiazate is determined by reversed phase high performance liquid chromatography using UV detection at 220 nm and internal standard calibration.

- REAGENTS

- Fosthiazate reference standard of known content

- Dimethyl phthalate, 99.95 % w/w, internal standard

Acetonitrile, HPLC grade

Internal standard solution. Dissolve (to the nearest 0.1 mg) 280 mg of dimethyl phthalate with acetonitrile in volumetric flasks (100 ml) using an ultrasonic bath. Ensure that a sufficient quantity of this solution is prepared for all samples and standards to be analysed.

Calibration solutions. Weigh in duplicate (to the nearest 0.1 mg) 24 mg of fosthiazate reference standard (s mg) into separate volumetric flasks (100 ml). Pipette internal standard solution (5.0 ml) into each flask, add acetonitrile (about 75 ml) and sonicate until complete dissolution. Allow the solutions to cool to ambient temperature and fill to the mark with acetonitrile. Mix well (calibration solutions CA and CB).

###### APPARATUS

High performance liquid chromatograph equipped with a detector suitable for operation at 220 nm (UV-detection) and an injection system capable of injecting 10 µl

Liquid chromatographic column stainless steel, 125 x 4 mm i.d., Agilent Hypersil ODS C18,      5 µm, or equivalent with the same selectivity

Electronic integrator or data system

Ultrasonic bath

Rotary shaker

Centrifuge apparatus

PROCEDURE

- Chromatographic conditions (typical)

Column temperature	25°C

Flow rate	1 ml/min

Detector wavelength	220 nm

Injection volume	10 µl

Retention time       	dimethyl phthalate (internal standard) approximately 4.7 min

fosthiazate approximately 6.4 min

Mobile phases	A: acetonitrile – water (1 – 2 v/v)

B: acetonitrile

Gradient program	Time	Eluent A	Eluent B

[ min ]	[ % ]	[ % ]

0	100	0

8	100	0

8.5	0	100			14	0	                  100 (stop time)

(Post time) 3 min	100	0

(b) Equilibration of the system. Pump sufficient mobile phase (use gradient programme) through the column to equilibrate the system. Inject 10 l portions of the calibration solution CA and repeat the injections until retention times and peak areas deviate by less than            0.5 % from the mean for three successive injections.

- (c) Sample preparation. Prepare sample solutions in duplicate for each sample. Weigh (to the nearest 0.1 mg) sufficient sample (w mg) to contain about 24 mg of fosthiazate (s mg) into a volumetric flask (100 ml). Pipette internal standard solution (5.0 ml), add acetonitrile (about 75 ml) and sonicate until complete dissolution. Allow the solutions to cool to ambient temperature and fill to the mark with acetonitrile. Mix thoroughly. Filter an aliquot of each prepared solution through a 0.45 µm PTFE filter prior to analysis (sample solutions S1 and S2).

(d) Determination.

Inject 10 l portions of the second calibration solution (CB) for two successive injections. The mean response factor for this solution should deviate by no more than 1% from those for the first calibration solution (CA) (see paragraph (b) Equilibration of the system), otherwise the calibration solutions should be prepared again.

Inject in duplicate 10 l portions of each sample solution (S1, S2, …,etc.) bracketing them by single injections of calibration solutions (CA and CB) using the following sequence:

- CA, S1, S1, S2, S2, CB, S3, S3, S4, S4, CA…

(e) 	Calculation. Determine the peak area of fosthiazate and calculate the mean value of response factors from the calibration solutions bracketing the injections of the sample solutions and use this value for calculating the fosthiazate content of the bracketed sample solutions. The fosthiazate content is the mean value of two sample solutions.

- fi=Ir×s×PHs

- fosthiazate content =f×HwIq×w [g/kg] (M)

where:

fi	= individual response factor

f	= mean response factor

Hs	= peak area of fosthiazate in the calibration solution

Hw	= peak area of fosthiazate in the sample solution

Ir	= peak area of the internal standard in the calibration solution

Iq	= peak area of the internal standard in the sample solution

s	= mass of fosthiazate reference standard in the calibration solution (mg)

w   = mass of sample taken (mg)

P	= purity of fosthiazate reference standard (g/kg)

Typical Chromatograms 585/TC/M/3

- Sample solution of fosthiazate technical without internal standard solution

- Calibration solution of fosthiazate

- Sample solution of fosthiazate technical with internal standard solution

FOSTHIAZATE GRANULES

- 585/GR/M/-

1. Sampling. Take at least 100 g.

2. Identity test

2.1 HPLC. As for fosthiazate technical 585/G/M/2.1

#### 3. Fosthiazate

#### As for fosthiazate technical 585/GR/M/3 except

- Sample preparation. Prepare sample solutions in duplicate for each sample. Weigh (to the nearest 0.1 mg) sufficient sample (w mg) to contain about 24 mg of fosthiazate (s mg) into a volumetric flask (100 ml). Pipette internal standard solution (5.0 ml), add acetonitrile (about 75 ml) and sonicate for minimum 15 minutes. Then place the flask in a rotary shaker for 30 minutes. Allow the solutions to cool to ambient temperature and fill to the mark with acetonitrile. Mix thoroughly. Transfer into a centrifuge tube and centrifuge at maximum 2000 rpm for 10 min. Filter an aliquot of each prepared solution through a 0.45 µm PTFE filter prior to analysis (sample solutions S1 and S2).

###### Typical Chromatograms 585/GR/M/-

###### Blank sample solution

- Sample solution of Fosthiazate 10 G without internal standard solution

- Calibration solution

- Sample solution of Fosthiazate 10 G with internal standard solution