Etpyrafen

997

HPLC method

CIPAC Full Scale Collaborative Trial

5191/m

By Ms Haixia Wang

Shenyang Research Institute of Chemical Industry Co., Ltd

Pesticides Test Laboratory

No.8, Shenliao Dong Road, Tiexi District, Shenyang City,

P. R. of China

Presented at the CIPAC meeting in Braunschweig, Germany

June, 2019

### ETPYRAFEN

- ISO common name 	Etpyrafen

Chemical name    	(Z)-2-(4-tert-butylphenyl)-2-cyano-1-(1-ethyl-3-methylpyrazol-5-yl)vinyl 2,2-dimethylpropionate (IUPAC)

(1Z)-2-cyano-2-[4-(1,1-dimethylethyl)phenyl]-1-(1-ethyl-3-methyl-1H-pyrazol-5-yl)ethenyl 2,2-dimethylpropanoate (CA, 1253429-01-4)

Empirical formula 	C24H31N3O2

RMM  	393.53

m.p.	97.2°C~98.6℃

v.p.	8.9×10-8 Pa at 20°C

Solubility                 In water 0.0890 mg/L at 20°C; In methanol 114~133 g/L, hexane 29~33 g/L. In acetone, dichloromethane, xylene, ethyl acetate >250 g/L

Stability	Stable for 14 d at 54 ℃

Description	The pure material is a white, odourless solid

Formulation		Suspension concentrates (SC)

### ETPYRAFEN technical

997/TC/(M)/-

1 Sampling. Take at least 100 g.

2. Identity test

2.1 HPLC. Use the reversed phase HPLC method below. The relative retention time of the etpyrafen peak in the sample solution should not deviate by more than 1.5% from that of the calibration solution.

2.2 Infrared. Prepare potassium bromide discs for the technical sample and etpyrafen reference substance. A typical potassium bromide disc should contain a sample prepared in the 0.15-0.35% by weight range. Scan the discs from 4000 to 400 cm-1. The spectrum of the sample should not differ significantly from that of the reference substance.

- 3 Etpyrafen

OUTLINE OF METHOD Etpyrafen is determined by reversed phase high performance liquid chromatography using UV detection at 230 nm and external standardisation.

- REAGENTS

Acetonitrile  HPLC grade

Phosphoric acid  Analytically pure

Water  HPLC grade

0.05% Phosphoric acid solution  Dissolve 0.5 ml Phosphoric acid into 1000 ml water

- Etpyrafen  reference standard of known content. Store refrigerated.

Calibration solutions. Weigh in duplicate about 40 mg (to the nearest 0.1 mg) of etpyrafen reference standard (s mg) into separate brown volumetric flasks (100 ml). Add acetonitrile (about 80 ml) and place the flask in an ultrasonic bath for 3 min. Allow to cool to ambient temperature and fill to the mark with acetonitrile. Mix thoroughly. (calibration solutions CA and CB).

- APPARATUS

High performance liquid chromatograph  equipped with a detector suitable for operation at 230 nm (UV-detection) and an injection system capable of injecting 5 µl.

Liquid chromatographic column  stainless steel, 150 x 4.6 mm (i.d.), Agilent ZORBAX SB C18, 5μm, or equivalent with the same selectivity.

Electronic integrator or data system

Ultrasonic bath

PROCEDURE

- Chromatographic conditions (typical)

Column temperature	30°C

Flow rate	1.0 ml/min

Detector wavelength	230 nm

Injection volume	5 µl

Mobile phase	  acetonitrile -  phosphoric acid solution, 800 + 200 (v/v)

Retention time       	etpyrafen approximately 8.3 min

- Equilibration of the system. Pump sufficient mobile phase through the column to equilibrate the system. Inject 5 µl portion of calibration solution CA until the response obtained from two consecutive injections deviate by less than 1.5%. Then inject 5 µl portion of calibration solution CB. The response factor for this solution should not deviate by more than 1.5% from that for calibration solution CA, otherwise prepare new calibration solutions.

- Preparation of etpyrafen TC sample. Prepare sample solutions in duplicate for each sample. Weigh (to the nearest 0.1 mg) sufficient sample (w mg) to contain about 40 mg of etpyrafen into a brown volumetric flask (100 ml). Add acetonitrile (about 80 ml) and place the flask in an ultrasonic bath for 3 min. Allow to cool to ambient temperature and fill to the mark with acetonitrile. Mix thoroughly. (sample solutions S1 and S2).

- Determination. Inject in duplicate 5 l portions of each sample solution bracketing them by injections of the calibration solutions as follows: calibration solution CA, sample solution S1, sample solution S1, calibration solution CB, sample solution S2, sample solution S2, calibration solution CA, and so on. Measure the relevant peak areas. Average the values of the duplicate sample injections. Calculate the mean values of the response factors of the calibration solution bracketing two sample solutions and use this value to calculate the etpyrafen content of the bracketed samples.

- Calculation. Determine the peak area of etpyrafen and calculate the mean value of response factors from the calibration solutions bracketing the injections of the sample solutions and use this value for calculating the etpyrafen content of the bracketed sample solutions. The etpyrafen content is the mean value of two sample solutions.

- fi=s×PHs

- Etpyrafen content =f×Hww g/kg

where:

fi	= individual response factor

f	= mean response factor

Hs	= peak area of etpyrafen in the calibration solution

Hw	= peak area of etpyrafen in the sample solution

s	= mass of etpyrafen reference standard in the calibration solution (mg)

w   = mass of sample taken (mg)

P	= purity of etpyrafen reference standard (g/kg)

Repeatability r	= 12 to 14 g/kg at 982 g/kg active ingredient content

Reproducibility R	= 20 to 21 g/kg at 982 g/kg active ingredient content

ETPYRAFEN suspension concentrates

- 997/SC/M/-

1 Sampling. Take at least 600 ml.

2. Identity test

2.1 HPLC. As for etpyrafen technical 997/TC/M/2.1.

2.2 Infrared. Homogenise the sample by vigorous shaking. Weigh 0.5 g (to the nearest 0.1 mg) of etpyrafen SC sample into a glass bottle, add 10 mL acetonitrile, place in ultrasonic bath and shake for 5 min, filter the liquid into a beaker, and dry at 70℃ under vacuum for 2 h to become solid. Allow to cool to ambient temperature in desiccator. Proceed as for etpyrafen technical 997/TC/M/2.2.

3 Etpyrafen

As for etpyrafen technical 997/TC/M/3 except:

- Preparation of sample. Homogenise the sample by vigorous shaking. Prepare sample solutions in duplicate for each sample. Weigh (to the nearest 0.1 mg) sufficient sample (w mg) to contain about 40 mg of etpyrafen (s mg) into a brown volumetric flask (100 ml). Add water (about 5 ml) to emulsify the sample, then add acetonitrile (about 80 ml) and place the flask in an ultrasonic bath for 3 min. Allow to cool to ambient temperature and fill to the mark with acetonitrile. Mix thoroughly. Filter an aliquot of each prepared solution through a 0.45 µm PTFE filter prior to analysis (sample solutions S1 and S2).

Repeatability r	= 5.9 to 7.0 g/kg at 303 to 305 g/kg active ingredient content

Reproducibility R	= 10.1 to12.6 g/kg at 303 to 305 g/kg active ingredient content

4 Suspensibility

REAGENTS AND APPARATUS.  As for etpyrafen technical 997/TC/M/3 and MT 184.

PROCEDURE

- Preparation of suspension and determination of sedimentation. MT184.

- Determination of etpyrafen in the bottom 25 ml of suspension. After removal of the top 225 ml of suspension transfer the remaining 25 ml to a brown volumetric flask (100 ml) with acetonitrile (about 60 ml). Place the flask in an ultrasonic bath for 3 min. Allow to cool to ambient temperature and fill to the mark with acetonitrile. Mix thoroughly. Filter through a 0.45 µm filter prior to analysis. Determine the mass of etpyrafen (Q g) by 997/TC/M/3.

- Calculation

Suspensibility=111(c−Q)c%

where:

c    = mass of etpyrafen in the sample taken for the preparation of the suspension (g)

Q   = mass of etpyrafen in the bottom 25 ml of suspension (g)

Fig.1 Infrared spectrum of etpyrafen

Fig.2 HPLC chromatogram of etpyrafen standard

Fig.3 HPLC chromatogram of etpyrafen TC

Fig.4 HPLC chromatogram of etpyrafen SC