Chlorpyrifos

221

HPLC  method

CIPAC Full Scale Collaborative Trial

Shenyang SYRICIT Testing Co. Ltd.

### CHLORPYRIFOS

221

- ISO common name 	Chlorpyrifos

Chemical name    	O,O-diethyl O-3,5,6-trichloro-2-pyridyl phosphorothioate(IUPAC)

O,O-diethyl O-(3,5,6-trichloro-2-pyridyl) phosphorothioate(CA; 2912-88-2)

Empirical formula 	C9H11Cl3NO3PS

RMM  	350.6

m.p.	42-43.5°C

v.p.	27mPa at 25°C

Solubility                     In water 1.4 mg/L at 25°C; In bezene 7900,actone 6500,chloroform 6300, carbon disulphide 5900, diethyl ether 5100,xylene 5000, isoctanol 790, methanol 450(all in g/kg, 25°C )

Stability	Rate of hydrolysis increases with pH, and in the presence of copper and possibly of other metals that can form chelates; DT50 1.5d(water, pH8, 25°C) to 100d (phosphate buffer, pH7, 15°C)

Description	The pure material is colourless crystals, with a mild mercaptan odour

Formulation		Emulsifiable concentrates (EC)

### CHLORPYRIFOS technical

221/TC/M/-

1 Sampling. Take at least 100 g.

2. Identity test

2.1 HPLC. Use the reversed phase HPLC method below. The relative retention time of the Chlorpyrifos peak in the sample solution should not deviate by more than 1.5% from that of the calibration solution.

2.2 Infrared. Prepare potassium bromide discs for the technical sample and Chlorpyrifos reference substance. A typical potassium bromide disc should contain a sample prepared in the 0.15-0.35% by weight range. Scan the discs from 4000 to 600 cm-1. The spectrum from the sample should not differ significantly from that of the reference substance.

2.3 GC-MS

PROCEDURE

- Preparation of sample.  Weigh (to the nearest 0.1 mg) sufficient sample to contain about  50 mg of Chlorpyrifos into a 10-mL glass bottle. Add chloroform  to dissolve it and mix thoroughly.

- Chromatographic conditions (typical)

Gas chromatographic column  quartz, 30m x 0.25 mm (i.d.), coated with diphenyl polysiloxane 95/5(e.g. TG-1701MS), film thickness 0.25µm

Injection system

Injetor	Split injection

Split ratio                                20:1

Gas flow(He) rate(ml/min)      2.0

Temperature

Oven program                         80℃ hold for 3min

Gradient 10℃/min to 180℃,hold for 5min, 30℃/min to 250℃,hold for 5min

Injector                                     260

Injection volume                       1µl

Ion source                                  EI, 70ev

Source temperature                   280℃

Quad temperature                    160℃

Range of mass number              30～700

Run time                                   25.3 min

Retention times                         Chlorpyrifos  about 19.8 min

The spectrum produced from the sample should not differ significantly from that of the standard.

- 3 Chlorpyrifos

OUTLINE OF METHOD Chlorpyrifos is determined by reversed phase high performance liquid chromatography using UV detection at 290 nm and external standardisation.

- REAGENTS

Acetonitrile  HPLC grade

Glacial acetic acid HPLC grade

Water  HPLC grade

- Chlorpyrifos  standard of known content. Store refrigerated.

Calibration solutions. Weigh in duplicate about 50 mg (to the nearest 0.1 mg) of Chlorpyrifos  standard (s mg) into separate volumetric flasks (100 ml). Add acetonitrile to the mark and place the flask in an ultrasonic bath for 5 min. Allow to cool to ambient temperature. Mix thoroughly. (calibration solutions CA and CB).

- APPARATUS

High performance liquid chromatograph  equipped with a detector suitable for operation at 290 nm (UV-detection) and an injection system capable of injecting 5 µl.

Liquid chromatographic column  stainless steel, 250 x 4.6 mm (i.d.), Agilent Extend-C18, 5 µm, or equivalent with the same selectivity.

Electronic integrator or data system

Ultrasonic bath

PROCEDURE

- Chromatographic conditions (typical)

Column temperature	25℃

Flow rate	1.0 ml/min

Detector wavelength	290 nm

Injection volume	5 µl

Mobile phase	acetonitrile – water – glacial acetic acid , 820 + 175 + 5 (v/v)

Retention time       	Chlorpyrifos approximately 8.1 min

- Equilibration of the system. Pump sufficient mobile phase through the column to equilibrate the system. Inject 5 µl portion of calibration solution CA until the response obtained from two consecutive injections deviate by less than 1.0%.

- Preparation of sample. Prepare sample solutions in duplicate for each sample. Weigh (to the nearest 0.1 mg) sufficient sample (w mg) to contain about 50mg of Chlorpyrifos (s mg) into a volumetric flask (100 ml). Add acetonitrile to the mark and place the flask in an ultrasonic bath for 5 min. Allow to cool to ambient temperature. Mix thoroughly. (sample solutions S1 and S2).

- Determination. Inject 5 µl portion of calibration solution CB. The response factor for this solution should not deviate by more than 1.0% from that for calibration solution CA, otherwise prepare new calibration solutions. Inject in duplicate 5 l portions of each sample solution bracketing them by injections of the calibration solutions as follows: CA, S1, S1, CB, S2, S2, CA, and so on. Measure the relevant peak areas.

- Calculation. Determine the peak area of Chlorpyrifos and calculate the mean value of response factors from the calibration solutions bracketing the injections of the sample solutions and use this value for calculating the Chlorpyrifos content of the bracketed sample solutions. The Chlorpyrifos content is the mean value of two sample solutions.

- fi=s×PHs

- Chlorpyrifos content =f×Hww g/kg

where:

fi	= individual response factor

f	= mean response factor

Hs	= peak area of Chlorpyrifos in the calibration solution

Hw	= peak area of Chlorpyrifos in the sample solution

s	= mass of Chlorpyrifos  standard in the calibration solution (mg)

w   = mass of sample taken (mg)

P	= purity of Chlorpyrifos  standard (g/kg)

CHLORPYRIFOS EMULSIFIABLE concentrates

- 221/EC/M/-

1 Sampling. Take at least 100 ml.

2. Identity test

2.1 HPLC. As for Chlorpyrifos technical 221/TC/M2.1

2.2 GC-MS. As for Chlorpyrifos technical 221/TC/M2.3

3 Chlorpyrifos

As for Chlorpyrifos technical 221/TC/M except

- Preparation of sample.  Prepare sample solutions in duplicate for each sample. Weigh (to the nearest 0.1 mg) sufficient sample (w mg) to contain about 50 mg of Chlorpyrifos (s mg) into a volumetric flask (100 ml).  Add acetonitrile (about 80 ml) and place the flask in an ultrasonic bath for 5 min . Allow to cool to ambient temperature and fill to the mark with acetonitrile. Mix thoroughly.(sample solutions S1 and S2).

Fig.1 Infrared spectrum of Chlorpyrifos

Fig.2 HPLC chromatogram of Chlorpyrifos standard

Fig.3 HPLC chromatogram of Chlorpyrifos TC

Fig.4 HPLC chromatogram of Chlorpyrifos EC

Fig.5 GC-MS spectrum of  Chlorpyrifos  standard

Fig.6 GC-MS spectrum of  Chlorpyrifos TC

Fig.7 GC-MS spectrum of  Chlorpyrifos EC